COURSEWORK

Portfolio 2

SANDBOX APPLICATION

NAME: ARCHANA BINDHU

STUDENT ID: A000254426

- NETWORK DIAGRAM

STEPS:

Gateway Router:

- Router1> enable

- Router1# configure terminal

- Router1(config)# interface fastethernet 0/0

- Router1(config-if)# ip address 192.168.9.1 255.255.255.0

- Router1(config-if)# no shutdown

- Router1(config-if)# exit

- Router1(config)# interface fastethernet 0/1

- Router1(config-if)#ip address 192.168.109.1 255.255.255.0

- Router1(config-if)#no shutdown

- Router(config-if)#ex

Desktop (PC):

- Click the PC and open it then go to the desktop >IP Configuration>change to static> Set IP address.

- Set all PC for this.

- And ping 192.168.9.1

Screenshot for pinging PC to Gateway

Application Server:

- Click the Application Server and open then go to the desktop > IP Configuration>change to static>Set IP address.

- Click the Application Server and open then go to the service >Click HTTP>Turn on HTTP and>HTTPS > and HTML page in Application Server.

- Click the Application Server and open it then goes to the Desktop>Web Browser>Type the IP address 192.168.109.2

Screenshot for web server showing a web site in web browser

2. IP ADDRESS TABLE

| DEVICE | ROLE | IP ADDRESS | SUBNET MASK |
| --- | --- | --- | --- |
| Desktop VM (Ubuntu Desktop) | Management | 192.168.9.2 | 255.255.255.0 |
| Gateway Router VM (enpOs3) (Ubuntu Server) | Internet Access | 10.0.2.15 | 255.255.255.0 |
| Gateway Router VM (enpOs8) | Subnet 01- Internal Network | 192.168.9.1 | 255.255.255.0 |
| Gateway Router VM (enpOs9) (Ubuntu Server) | Subnet 02- Internal Network | 192.168.109.1 | 255.255.255.0 |
| Application Server VM (Bitnami) | Server | 192.168.109.2 | 255.255.255.0 |

3. GIT PAGES LAB REPORT

Github Project link:- https://archanasandbox.github.io/archana-sandbox/

Github File link:- https://github.com/archanasandbox/archana-sandbox.git

CONFIGRATION STEPS:

- You need to create three VM Machine’s for this project.

- Download the Virtual Box (VM) and install.

- Download the three OS :

- Ubuntu Desktop (.iso format) or our Desktop OS like ( kali, Linux, windows etc..)

- Ubuntu Server OS (.iso format)

- Application Server in Bitnami Wordpress (.ova format)

- Add the three OS in Virtual Box.

UBUNTU SERVER OS CONFIGURATION STEPS:

Step 1:

Create a new Virtual Machine (VM) for Ubuntu Server

- Open VirtualBox

- Click on new to create a new virtual machine.

- Name the VM ( eg; Ubuntu Server)

- Select the type as Linux and the version as Ubuntu ( 64-bit)

- Allocate memory ( RAM) for the VM (eg: 2048 or higher based on our systems capacity).

- Choose to create a virtual hard disk now and set a sufficient disk size ( eg: 10 GB)

- Click Create.

Step 2:

Configure the Network Interfaces

You need two network interfaces on the Ubuntu Server VM to act as a router between the two subnets:

- Go to Settings then select Network.

- On Adapter 1:

- Set to Internal Network (name it to intnet). This will be for subnet.

- Click the Advanced tab and set the adapter type to PCnet-FAST III or another supported type. Note: Within the ubuntu server terminal, this card will typically have a network adapter name of enp0s8.

- On Adapter 2:

- Enable this adapter and set it to another Internal Network (name it differently, e.g., intnet1). This will be for Subnet 2.

- Within the ubuntu server terminal, this card will typically have a network adapter name of enp0s9.

- On Adapter 3:

- Set to NAT. Note: Within the ubuntu server terminal, this card will typically have a network adapter name of enp0s3.

- Its IP address will be assigned via DHCP.

- It will be used to provide access to the internet via the host computer.

Step 3:

Install Ubuntu Server

- Start the VM and select the Ubuntu Server ISO as the boot disk.

- Go through the installation process.

- Set your time zone, keyboard layout etc.

- Create a user account and set a strong password.

- Then select the option to install Open SSH server

- Complete the installation and then reboot the VM.

Step 4:

Configure Static IPs on the Network Interfaces

- After installation we need to assign static IP addresses to both network interfaces (each in different subnets).

- Log in to the Ubuntu Server VM

- Edit the network configuration file:

- -  sudo nano /etc/netplan/00-installer-config.yaml

- Type the code.

- Save it (CTRL +X) > type YES > and then ENTER

- Apply the network change

- sudo netplan apply

- ip a

Step 5:

Enable IP Forwarding

To allow routing between the two subnets you need to enable IP forwarding.

- Open the sysctl configuration file:

- sudo nano /etc/sysctl.conf

- Uncomment the line or add it if it is not present

- net.ipv4_forward=1

- Apply the changes:

-  sudo sysctl -p

Step 6:

Set Up IPTables for Routing

You may also want to configure iptables to ensure packets are forwarded between the subnets.

- Configure iptables to allow forwarding:

❖ sudo iptables -A FORWARD -i enp0s3 -o enp0s8 -j ACCEPT

❖ sudo iptables -A FORWARD -i enp0s8 -o enp0s3 -j ACCEPT

- To make the changes permanent, you can save the iptables rules:

❖ sudo apt install iptables-persistent

❖ sudo netfilter-persistent save

❖ sudo netfilter-persistent reload

Ubuntu OS Configuration Steps:

Steps:

Installing Ubuntu Desktop in VM

To create a virtual machine and install a GUI-based OS from an ISO file

- Launch VirtualBox

- Click “New” to create a new machine

- Fill in the details as requested so that the new machine is:

❖ Named “Ubuntu Desktop”

❖ The ISO named “ubuntu-24.04.1-desktop-amd64.iso” is selected

❖ The checkbox for “Skip Unattended Installation” is checked

- Click “Next”. Now ensure the machine has:

❖ 2048mb of Base Memory

❖ Two processors

- Click “Next”. Create a Virtual Hard Disk for the machine. This will need to be a minimum of 25GB.

- Click “Next”. Verify your settings are as above and click “Finish”.

- Click “Start” to boot your virtual machine

- Select “Try or Install Ubuntu”

- Click “Install Ubuntu”

- You need to create an Ubuntu account and click a Restart Now.

Step 2:

Configure the Network Interfaces

You need one network interfaces on the Ubuntu Desktop VM.

- Go to Settings of your new VM.

- Select Network.

- Adapter 1: Set to Internal Network (name it differently, e.g., intnet).

- Open Ubuntu desktop in VM and login.

- Go to the settings > Network> enp0s3 setting> IPV4 > IPV4 Method change to (Manual)> Add address =192.168.22.2, Add netmask =255.255.255.0 or 24, Add gateway =192.168.22.1

- Apply

- Disconnect the Network and connect.

Bitnami Web Application Configuration Step:

To create a Bitnami virtual machine using VirtualBox

Step 1:

Install Bitnami Application in VM.

- Click “File” then “Import appliance”.

- Navigate to the file named “bitnami-wordpress-6.3.1-r0-debian-11-amd64.ova”.

- Click “Open”.

- Click “Next”.

- Click “Finish”.

- Click “Start” to launch the machine.

- The first-time log in details is displayed on-screen.

- You will be required to reset the password on your first log in.

- Close the Bitnami.

Step 2:

Configure the Network Interfaces

- Go to Settings of your new VM.

- Select Network.

- Adapter 1: Set to Internal Network (name it differently, e.g., intnet1).

- Open Bitnami Application in VM and login.

- Type the code

❖ sudo nano /etc/network/interfaces

❖ auto enp0s3

iface enp0s3 inet static

address 192.168.122.2

netmask 255.255.255.0

gateway 192.168.122.1

❖ Save it (CTRL + X) type (yes) and enter.

❖ sudo ifdown enp0s3 && sudo ifup enp0s3

❖ sudo systemctl restart networking

- ip a

4.FUNCTIONAL TEST RESULTS

Evidence that all the VM’s can communicate as per the design ( eg ; ping results, screenshots of application access).

Screenshots for Functional Test Results :

Ubuntu Server OS:

- Network IP configuration for Ubuntu Server:

- Ping Ubuntu Server to Ubuntu Desktop Using IP address 192.168.9.2

- Ping Ubuntu Server to Ubuntu Desktop Using IP address 192.168.109.2

Ubuntu Desktop OS:

- Network IP Configuration for Ubuntu Desktop

- Ping Ubuntu Desktop to Ubuntu Server Using IP address 192.168.9.1.

BITNAMI APPLICATION SERVER:

- Network IP configuration for Bitnami Application Server

- Ping Bitnami Application Server to Ubuntu Server Using IP address 192.168.109.1

Screen Recording:

https://youtu.be/Bce0Qz-iDj4